ZIMOX RFID Tool User Guid

## 1. Version Information

Software Version: v3.0.5.0

Terminal App Version: v1.1.2.0

## 2. Download and Installation

Download Link: https://github.com/blu-blue/RFIDToolsForThailand

## 3. Read a tag

1. Connect desktop reader and computer via the usb cable.

2. Put one tag on the desktop reader. Like this

3. Chose the driver’s com number, select the baudrate to 115200 , then click the connect button.

4. Click Start reading button, the tag number will be shown in the text box

## 4. Import asset information

1. Click Open Directory Button to access the data folder , find “InputAssetsTable.xlsx” and Open “InputAssetsTable.xlsx” in Excel. Add some data to this spreadsheet, then close it.

2. Navigate to “Query the database” panel, click import button. Wait the importing  progress finish then click Complete button.

## 5. Bind a tag with one asset data

## 6. Send assets table to terminal

After connecting the handheld terminal to the PC via USB, refresh the terminal and click PC -> Terminal

Click Confirm button

Copy PCAssetsTable.xlsx to the terminal's download directory

## 7. Scan using the terminal

Wait Reader status from Unconnected to Connected

Click Load Assets button, choose the PCAssetsTable.xlsx

Pull the back trigger to start scanning,

The table will automatically export to the download folder after you release the trigger.

## 8. Get terminal files on PC

Click Terminal -> PC button

Click Open directory button , Inventory Results Table is targetFile.xlsx